SECTION 077123

GUTTERS AND DOWNSPOUTS

USE THIS SECTION FOR RESIDENTIAL TYPE, PREFORMED (OFF THE SHELF) GUTTERS.  USE COMMERCIAL VERSION FOR HEAVY DUTY GUTTERS.

PART 1   GENERAL

1.01	SUBMITTALS

A.	Product Data:  Catalog sheets, specifications, and installation instructions for the following:

1.	Gutter.

2.	Gutter hanger bracket.

3.	Downspout.

4.	Sealant.

PART 2   PRODUCTS

2.01	MATERIALS

A.	Gutter:  Free floating box or K style, .027 gage aluminum, 5 inch wide gutter in 10 foot maximum lengths.

B.	Fascia Cleat (Surface Mounted):  Gutter manufacturer's continuous aluminum fascia cleat designed for attachment directly to the fascia board.

SELECT ABOVE OR BELOW.  COORDINATE WITH DRAWINGS.

B.	Fascia Cleat (Roof Deck Mounted):  Gutter manufacturer's continuous aluminum fascia cleat designed for attachment to the roof deck.

C.	Gutter Hanger Brackets:  Gutter manufacturer's aluminum hanger brackets designed to span across the top of the gutter forming a spreader and stiffener bar.

D.	Downspout:  3 x 4 inch corrugated, .027 gage aluminum, in 8 or 10 foot lengths.

E.	Downspout Strap:  Downspout manufacturer's standard strap.

F.	Finish:

SELECT ONE OF THE FOLLOWING.

1.	Standard mill finish.

2.	White baked on enamel.

3.	Dark brown baked on enamel.

G.	Accessories:  Gutter manufacturer's standard or recommended sealant, pop rivets, screws, and bolts.

PART 3   EXECUTION

3.01	INSTALLATION

A.	Unless shown or specified otherwise, install the gutter system in accordance with the manufacturer's installation instructions.

B.	Install continuous fascia cleat.

SELECT ONE OF THE FOLLOWING.  COORDINATE WITH DRAWINGS.

1.	Secure the fascia cleat to the fascia board with aluminum nails 6 inches oc.

2.	Secure the fascia cleat to the roof deck with aluminum nails 6 inches oc.

C.	Trim the top front and back edges of one end of each gutter section so the sections will overlap each other 2 inches and nest snugly together.  Before joining sections together, apply sealant between the mating surfaces.

D.	Install gutter brackets 3 feet on center.

SELECT ONE OF THE FOLLOWING PARAGRAPHS.

1.	Lock the front of the bracket to the gutter edge and secure the other end to the fascia board with aluminum screws.

2.	Lock the front of the bracket to the gutter edge and secure the other end to the roof deck with aluminum screws.

E.	Install gutter end pieces, mitered corners, and drop tubes.  Apply sealant between mating surfaces and secure with pop rivets.

F.	Install downspouts with end joints that telescope a minimum of 1-1/2 inches.

G.	Install necessary offsets and elbows.

H.	Install a minimum of 2 straps at each downspout section.  Fasten downspouts to straps with pop rivets.

I.	Secure straps to masonry or concrete wall with machine bolts in lead shields.

J.	Secure straps to wood walls with screws.

K.	Discharge Elbows:  Fasten leader shoes to downspouts with pop rivets.

END OF SECTION